# ar_meta Cache Feature - Comprehensive Technical Documentation

# Table of Contents

[Overview](#overview)

[Problem Statement](#problem-statement)

[Solution Architecture](#solution-architecture)

[Kernel Changes](#kernel-changes)

[VDRV Interface Layer Changes](#vdrv-interface-layer-changes)

[Integration Points](#integration-points)

[Data Flow Analysis](#data-flow-analysis)

[API Reference](#api-reference)

[Usage Examples](#usage-examples)

[Performance Considerations](#performance-considerations)

[Future Extensions](#future-extensions)

[Related Files](#related-files)

[Appendix](#appendix)

──────────────────────────────────────────────────

# Overview

The `ar_meta` cache is an Arista-specific extension to the Linux kernel's `sk_buff` structure

that provides efficient per-packet metadata caching for wireless driver operations. This feature

enables the Arista wireless driver stack to cache critical packet metadata directly in the

socket buffer structure, eliminating the need for repeated expensive lookups to hardware

descriptors or control block structures during packet processing.

## Key Features

**Direct sk_buff Integration**: Metadata is stored directly in the kernel's sk_buff structure

**Zero-Copy Access**: No memory allocation or copying required for metadata access

**Lifecycle Management**: Automatic initialization and cleanup during skb operations

**Minimal Memory Footprint**: Only 2 bytes added to sk_buff structure

**Thread-Safe**: Follows kernel sk_buff locking semantics

──────────────────────────────────────────────────

# Problem Statement

## Background

In wireless networking, each packet carries metadata that is essential for proper processing:

**TID (Traffic Identifier)**: Determines QoS priority and queue selection (0-15)

**Peer ID**: Identifies the wireless client

**Rate Information**: MCS, NSS, bandwidth, etc.

## The Challenge

The Arista wireless driver stack processes packets through multiple layers:

┌─────────────────────────────────────────────────────────────────┐
│                     Packet Processing Flow                      │
├─────────────────────────────────────────────────────────────────┤
│                                                                 │
│  Hardware Rx ──► Vendor Driver ──► VDRV Layer ──► AR Core ──►  │
│                                                                 │
│  ──► QoS Processing ──► ACL Processing ──► Network Stack       │
│                                                                 │
└─────────────────────────────────────────────────────────────────┘

At each layer, packet metadata like TID may be needed. Previously, this required:

**QDF Control Block Access**: Using `QDF_NBUF_CB_RX_TID_VAL(skb)` macro

**Hardware Descriptor Queries**: Via `vdrv_dp_if_rx_msdu_get_rx_info()`

**Repeated Lookups**: Same metadata queried multiple times per packet

## Performance Impact

Each metadata lookup involves:

Memory indirection through control block structures

Potential cache misses due to scattered data

Function call overhead for abstraction layers

For high-throughput scenarios (1000s of packets/second), these overheads accumulate

significantly.

──────────────────────────────────────────────────

# Solution Architecture

## Design Principles

**Locality of Reference**: Store frequently accessed metadata in sk_buff itself

**Single Write, Multiple Read**: Cache metadata once, read many times

**Transparent Integration**: Existing code continues to work unchanged

**Backward Compatibility**: No changes required to vendor driver code

## Architecture Diagram

┌─────────────────────────────────────────────────────────────────────────┐
│                        ar_meta Cache Architecture                       │
├─────────────────────────────────────────────────────────────────────────┤
│                                                                         │
│  ┌──────────────────────────────────────────────────────────────────┐  │
│  │                         Linux Kernel                             │  │
│  │  ┌────────────────────────────────────────────────────────────┐  │  │
│  │  │                      struct sk_buff                        │  │  │
│  │  │  ┌──────────────────────────────────────────────────────┐  │  │  │
│  │  │  │  ... existing fields ...                             │  │  │  │
│  │  │  │  void *pkt_trace;        /* AR packet trace */       │  │  │  │
│  │  │  │  struct {                                             │  │  │  │
│  │  │  │      __u8 tid;           /* Cached TID (0-15) */     │  │  │  │
│  │  │  │      __u8 reserve;       /* Reserved for future */   │  │  │  │
│  │  │  │  } ar_meta;              /* NEW: Arista metadata */  │  │  │  │
│  │  │  │  struct skb_ext *extensions;                         │  │  │  │
│  │  │  │  ... remaining fields ...                             │  │  │  │
│  │  │  └──────────────────────────────────────────────────────┘  │  │  │
│  │  └────────────────────────────────────────────────────────────┘  │  │
│  └──────────────────────────────────────────────────────────────────┘  │
│                                    │                                    │
│                                    ▼                                    │
│  ┌──────────────────────────────────────────────────────────────────┐  │
│  │                    VDRV Interface Layer                          │  │
│  │  ┌────────────────────────────────────────────────────────────┐  │  │
│  │  │  vdrv_dp_if_ar_meta_get_tid()    - Read cached TID        │  │  │
│  │  │  vdrv_dp_if_ar_meta_set_tid()    - Write TID to cache     │  │  │
│  │  │  vdrv_dp_if_ar_meta_get_reserve() - Read reserved field   │  │  │
│  │  │  vdrv_dp_if_ar_meta_set_reserve() - Write reserved field  │  │  │
│  │  │  vdrv_dp_if_ar_meta_clear()       - Clear all ar_meta     │  │  │
│  │  └────────────────────────────────────────────────────────────┘  │  │
│  └──────────────────────────────────────────────────────────────────┘  │
│                                    │                                    │
│                                    ▼                                    │
│  ┌──────────────────────────────────────────────────────────────────┐  │
│  │                      AR Core Driver                              │  │
│  │  ┌────────────────────────────────────────────────────────────┐  │  │
│  │  │  ar_dp_rx_handle()      - Main Rx processing              │  │  │
│  │  │  ar_dp_tx_handle()      - Main Tx processing              │  │  │
│  │  │  ar_qos_dp_rx_set_prio() - QoS priority setting           │  │  │
│  │  │  ar_bgmon.c             - Background monitor processing   │  │  │
│  │  └────────────────────────────────────────────────────────────┘  │  │
│  └──────────────────────────────────────────────────────────────────┘  │
│                                                                         │
└─────────────────────────────────────────────────────────────────────────┘

──────────────────────────────────────────────────

# Kernel Changes

## Patch File Location

ap/platform/patches/kernel/5.4/12.5/common/ar_meta_cache.patch

## Patch Summary

The kernel patch modifies three files to add and manage the `ar_meta` structure:

| File | Function | Change Description |

|------|----------|-------------------|

| `include/linux/skbuff.h` | struct sk_buff | Add `ar_meta` struct with `tid` and `reserve` fields |

| `net/core/skbuff.c` | `__build_skb()` | Initialize ar_meta to zero when building new skb |

| `net/core/skbuff.c` | `__skb_clone()` | Copy ar_meta from source to cloned skb |

| `net/core/skbuff.c` | `skb_clone()` | Initialize ar_meta to zero for fclone case |

| `net/core/skbuff.c` | `skb_init()` | Log ar_meta support enabled at boot |

| `net/core/skbuff_recycle.c` | `skb_recycler_clear_flags()` | Clear ar_meta when recycling skb |

## Detailed Patch Analysis

### 1. Structure Definition (skbuff.h)

// Location: include/linux/skbuff.h, after pkt_trace field
struct {
  __u8    tid;      /* Traffic Identifier (0-15) */
  __u8    reserve;  /* Reserved for future use */
} ar_meta;

**Why this location?**

Placed after `pkt_trace` field (another Arista extension)

Before `skb_ext *extensions` to maintain structure alignment

Grouped with other Arista-specific fields for clarity

**Memory Impact:**

Adds 2 bytes to sk_buff structure

Minimal impact due to structure padding in most cases

### 2. Initialization in __build_skb() (skbuff.c)

// Location: net/core/skbuff.c, in __build_skb() function
#endif
+   skb->ar_meta.tid = 0;
+   skb->ar_meta.reserve = 0;

return __build_skb_around(skb, data, frag_size);

**Why here?**

`__build_skb()` is the low-level function for creating skb from data buffer

Called by `build_skb()` and other allocation paths

Ensures ar_meta is initialized for all newly built skbs

### 3. Clone Handling in __skb_clone() (skbuff.c)

// Location: net/core/skbuff.c, in __skb_clone() function
C(head_frag);
C(data);
C(truesize);
+   C(ar_meta);
refcount_set(&n->users, 1);

**Why copy ar_meta during clone?**

`__skb_clone()` creates a clone that shares data with original

Metadata should be preserved as clone represents same packet

Uses existing `C()` macro for consistent field copying

### 4. fclone Case in skb_clone() (skbuff.c)

// Location: net/core/skbuff.c, in skb_clone() function
#ifdef CONFIG_AR_PKT_TRACE_ENABLE
n->pkt_trace = NULL;
#endif
+       n->ar_meta.tid = 0;
+       n->ar_meta.reserve = 0;
n->fclone = SKB_FCLONE_UNAVAILABLE;

**Why initialize to zero here?**

fclone (fast clone) path allocates from pre-allocated pool

These skbs may have stale data from previous use

Must explicitly clear ar_meta to prevent data leakage

### 5. Boot Message in skb_init() (skbuff.c)

// Location: net/core/skbuff.c, in skb_init() function
skb_extensions_init();
skb_recycler_init();
+
  +   pr_info("sk_buff ar_meta support enabled (tid: 8-bit, reserve: 8-bit)\n");

**Why add boot message?**

Confirms ar_meta feature is compiled into kernel

Aids debugging and verification

Documents field sizes for reference

### 6. Recycler Cleanup (skbuff_recycle.c)

// Location: net/core/skbuff_recycle.c, in skb_recycler_clear_flags()
skb->recycled_for_ds = 0;
skb->fast_qdisc = 0;
skb->int_pri = 0;
+   skb->ar_meta.tid = 0;
+   skb->ar_meta.reserve = 0;

**Why clear in recycler?**

SKB recycler reuses skbs for performance

Must clear all packet-specific data before reuse

Prevents metadata from previous packet affecting new packet

──────────────────────────────────────────────────

# VDRV Interface Layer Changes

## Why VDRV Layer?

The VDRV (Vendor Driver) interface layer serves as the abstraction between:

Arista core driver code (`ar/core/`)

Vendor-specific driver implementations (`QCA/licensed/`)

Adding ar_meta accessors here provides:

**Abstraction**: Core driver doesn't directly access kernel structures

**Portability**: Same API works across different kernel versions

**Maintainability**: Single point of change for ar_meta access patterns

## Files Modified

### Header File: `ap/src/wlan-drivers/ar/vdrv_if/inc/vdrv_dp_if.h`

Added function declarations and documentation block:

/*
 * ar_meta cache accessor functions
 *
 * These functions provide access to the ar_meta structure added to sk_buff
 * for caching Arista-specific packet metadata. The ar_meta structure is added
 * to the kernel's sk_buff structure (see ar_meta_cache.patch) and contains:
 *   - tid: 8-bit TID (Traffic Identifier) cache
 *   - reserve: 8-bit reserved field for future use
 *
 * This allows efficient caching and retrieval of per-packet metadata without
 * repeatedly querying hardware descriptors or control block structures.
 */

uint8_t vdrv_dp_if_ar_meta_get_tid(struct sk_buff* skb);
void vdrv_dp_if_ar_meta_set_tid(struct sk_buff* skb, uint8_t tid);
uint8_t vdrv_dp_if_ar_meta_get_reserve(struct sk_buff* skb);
void vdrv_dp_if_ar_meta_set_reserve(struct sk_buff* skb, uint8_t reserve);
void vdrv_dp_if_ar_meta_clear(struct sk_buff* skb);

### Implementation File: `ap/src/wlan-drivers/ar/vdrv_if/qca/common/vdrv_dp_if.c`

Added function implementations:

/*
 * ar_meta cache accessor functions
 *
 * These functions provide access to the ar_meta structure added to sk_buff
 * for caching Arista-specific metadata. The ar_meta structure contains:
 *   - tid: 8-bit TID (Traffic Identifier) cache
 *   - reserve: 8-bit reserved field for future use
 *
 * This allows the driver to cache and retrieve per-packet metadata efficiently
 * without repeatedly querying the hardware or control block structures.
 */

uint8_t vdrv_dp_if_ar_meta_get_tid(struct sk_buff* skb) { return skb->ar_meta.tid; }

void vdrv_dp_if_ar_meta_set_tid(struct sk_buff* skb, uint8_t tid) { skb->ar_meta.tid = tid; }

uint8_t vdrv_dp_if_ar_meta_get_reserve(struct sk_buff* skb) { return skb->ar_meta.reserve; }

void vdrv_dp_if_ar_meta_set_reserve(struct sk_buff* skb, uint8_t reserve) { skb->ar_meta.reserve = reserve; }

void vdrv_dp_if_ar_meta_clear(struct sk_buff* skb)
{
  skb->ar_meta.tid = 0;
  skb->ar_meta.reserve = 0;
}

## Placement Rationale

The functions are placed after `vdrv_dp_if_wbuf_set_priority()` because:

**Logical Grouping**: Near other TID/priority-related functions

**Similar Purpose**: All deal with packet metadata manipulation

**Code Organization**: Maintains consistent file structure

## CB-Independent TID Access Solution

This section documents the **CB-independent** solution where `ar_meta` becomes the

**primary** source of TID, completely eliminating dependency on the QDF control block (CB).

### Architecture Overview

┌─────────────────────────────────────────────────────────────────────────┐
│                    CB-INDEPENDENT TID FLOW                              │
├─────────────────────────────────────────────────────────────────────────┤
│                                                                         │
│  ┌───────────────────┐    TID extracted    ┌────────────────────┐      │
│  │ Hardware Rx       │──────────────────────│ Vendor Driver      │      │
│  │ Descriptor        │   from HAL/HW        │ (li/be/rh)         │      │
│  └───────────────────┘                      └─────────┬──────────┘      │
│                                                       │                 │
│                                                       │ DP_RX_TID_SAVE  │
│                                                       │ DP_RX_TID_SAVE  │
│                                                       │ _AR_META()      │
│                                                       ▼                 │
│                                            ┌────────────────────┐      │
│                                            │   skb->ar_meta.tid │      │
│                                            │   (PRIMARY STORAGE)│      │
│                                            └─────────┬──────────┘      │
│                                                       │                 │
│         ┌─────────────────────────────────────────────┼──────────────┐  │
│         │                                             │              │  │
│         ▼                                             ▼              ▼  │
│  ┌──────────────┐                          ┌──────────────┐  ┌────────┐ │
│  │vdrv_dp_rx_tid│                          │ar_qos_dp_rx_ │  │ ar_dp_ │ │
│  │  (reads from │                          │set_prio()    │  │rx_     │ │
│  │   ar_meta)   │                          │              │  │handle()│ │
│  └──────────────┘                          └──────────────┘  └────────┘ │
│                                                                         │
│  NOTE: CB (skb->cb) is NO LONGER used for TID read operations           │
│                                                                         │
└─────────────────────────────────────────────────────────────────────────┘

### Key Changes for CB-Independence

#### 1. New Macro: DP_RX_TID_SAVE_AR_META

**File:** `ap/src/wlan-drivers/QCA/licensed/spf12_5_cs/cmn_dev/dp/wifi3.0/dp_rx.h`

/*
 * DP_RX_TID_SAVE_AR_META - Save TID directly in ar_meta (CB-independent)
 *
 * This macro stores the TID value directly in the sk_buff's ar_meta structure,
 * bypassing the QDF control block (CB). This provides a CB-independent path
 * for TID access, improving performance and reducing coupling to the QCA
 * CB structure.
 */
#define DP_RX_TID_SAVE_AR_META(_nbuf, _tid) \
do { \
  (_nbuf)->ar_meta.tid = (_tid); \
} while (0)

**Why This Macro:**

Sets TID directly in `ar_meta` at the source (vendor driver)

Called right after TID is extracted from hardware descriptors

Makes `ar_meta` the primary TID storage, not a cache of CB

#### 2. Vendor Driver Rx Paths Modified

**Files:**

`ap/src/wlan-drivers/QCA/licensed/spf12_5_cs/cmn_dev/dp/wifi3.0/li/dp_li_rx.c`

`ap/src/wlan-drivers/QCA/licensed/spf12_5_cs/cmn_dev/dp/wifi3.0/be/dp_be_rx.c`

`ap/src/wlan-drivers/QCA/licensed/spf12_5_cs/cmn_dev/dp/wifi3.0/rh/dp_rh_rx.c`

**Change Applied to All Three:**

DP_RX_TID_SAVE(nbuf, tid);
/* Store TID in ar_meta for CB-independent access */
DP_RX_TID_SAVE_AR_META(nbuf, tid);

**Why These Locations:**

**Lithium (li)**: Line 651 - Main Rx processing loop

**Beryllium (be)**: Line 840 - Main Rx processing loop

**Rhine (rh)**: Line 1050 - Main Rx processing loop

These are the earliest points after TID is extracted from hardware

Ensures all downstream code has access to TID via ar_meta

#### 3. vdrv_dp_rx_tid() - Now Reads from ar_meta (CB-Independent)

**File:** `ap/src/wlan-drivers/ar/vdrv_if/qca/common/vdrv_dp_if.c`

**Before (CB-dependent):**

uint8_t vdrv_dp_rx_tid(struct sk_buff* skb) { return QDF_NBUF_CB_RX_TID_VAL(skb); }

**After (CB-independent):**

/*
 * vdrv_dp_rx_tid - Get TID from ar_meta (CB-independent)
 *
 * This function reads TID directly from the ar_meta structure, which is
 * populated by DP_RX_TID_SAVE_AR_META() in the vendor driver Rx path.
 * This provides a CB-independent path for TID access.
 */
uint8_t vdrv_dp_rx_tid(struct sk_buff* skb)
{
  /* Read TID from ar_meta - CB-independent */
  return skb->ar_meta.tid;
}

**Why This Change:**

**No CB dependency**: Reads directly from `ar_meta.tid`

**Faster access**: Simple struct member access vs CB macro expansion

**Clone-safe**: `ar_meta` is preserved during `skb_clone()` (kernel patch)

**Reliable**: Works even if CB is repurposed or cleared by other code

#### 4. vdrv_dp_if_wbuf_set_tid() - Primary ar_meta, CB for Backward Compatibility

**File:** `ap/src/wlan-drivers/ar/vdrv_if/qca/common/vdrv_dp_if.c`

/*
 * vdrv_dp_if_wbuf_set_tid - Set TID in ar_meta (CB-independent)
 *
 * This function sets TID in both ar_meta (primary) and CB (for backward
 * compatibility with any code that still reads from CB).
 */
void vdrv_dp_if_wbuf_set_tid(struct sk_buff* skb, int tid)
{
  /* Set TID in ar_meta (primary storage - CB-independent) */
  skb->ar_meta.tid = (uint8_t)tid;
  /* Also set in CB for backward compatibility with legacy code paths */
  wbuf_set_tid(skb, tid);
}

**Why This Design:**

`ar_meta.tid` is set **first** (primary storage)

CB is still updated for backward compatibility

Once all TID reads migrate to ar_meta, CB update can be removed

#### 5. ar_dp_rx_handle() - Uses ar_meta-aware Functions

**File:** `ap/src/wlan-drivers/ar/core/src/ar_dp.c`

if (qdf_unlikely(vdev->apc.enable || ...)) {
  // ... descriptor path ...
  tid = ars->tid;
  peer_id = ars->peer_id;
  /* Cache TID in ar_meta for efficient subsequent access without CB lookup */
  vdrv_dp_if_ar_meta_set_tid(skb, tid);
} else {
  status = AR_STATUS_SUCCESS;
  tid = vdrv_dp_rx_tid(skb);  /* Now reads from ar_meta (CB-independent) */
  peer_id = vdrv_dp_rx_peer_id(skb);
}

**Why This Design:**

In the descriptor path, TID comes from `ars->tid` (not CB)

Must explicitly set `ar_meta.tid` for this path

In the fast path, `vdrv_dp_rx_tid()` reads directly from ar_meta (CB-independent)

#### 6. ar_qos_dp_rx_set_prio() - Updates Effective TID in ar_meta

**File:** `ap/src/wlan-drivers/ar/core/src/ar_qos.c`

void ar_qos_dp_rx_set_prio(struct sk_buff* skb, struct ar_dp_vdev_s* vdev, uint8_t tid)
{
  uint8_t effective_tid;

  if (AR_IS_QOS_PRIO_FIXED(vdev)) {
    effective_tid = WME_AC_TO_TID(AR_GET_QOS_PRIO(vdev));
    ar_os_skb_set_priority(skb, effective_tid);
  } else {
    AR_CEIL_QOS_TID(vdev, tid);
    effective_tid = tid;
    ar_os_skb_set_priority(skb, tid);
  }
  /* Update effective TID in ar_meta (may differ from original due to QoS) */
  vdrv_dp_if_ar_meta_set_tid(skb, effective_tid);
}

**Why This Change:**

The effective TID may differ from the original TID (ceiling, fixed priority)

`ar_meta.tid` is updated to reflect the final effective TID

Downstream code always gets the correct processed value

### Benefits of CB-Independent Solution

| Benefit | Description |

|---------|-------------|

| **No CB Dependency** | TID access does not require QDF CB structure |

| **Primary Storage** | `ar_meta` is the authoritative source, not a cache |

| **Faster Access** | Direct struct member vs macro-expanded CB access |

| **Decoupled Design** | Arista code independent of QCA CB layout changes |

| **Clone-Safe** | `ar_meta` automatically copied during `skb_clone()` |

| **Future-Proof** | If CB structure changes, TID access is unaffected |

### Comparison: CB-Dependent vs CB-Independent

| Aspect | CB-Dependent (Old) | CB-Independent (New) |

|--------|---------------------|----------------------|

| TID Source | `skb->cb` via `QDF_NBUF_CB_RX_TID_VAL()` | `skb->ar_meta.tid` |

| Set Location | QCA code sets CB, Arista caches | QCA code sets ar_meta directly |

| Read Function | `vdrv_dp_rx_tid()` reads CB | `vdrv_dp_rx_tid()` reads ar_meta |

| Coupling | Tight coupling to QCA CB layout | Decoupled from CB |

| Performance | CB macro expansion overhead | Direct struct access |

| Clone Handling | CB may not be copied correctly | Kernel ensures ar_meta is copied |

### Files Modified for CB-Independence

| File | Change |

|------|--------|

| `dp_rx.h` | Added `DP_RX_TID_SAVE_AR_META` macro |

| `dp_li_rx.c` | Added `DP_RX_TID_SAVE_AR_META()` call |

| `dp_be_rx.c` | Added `DP_RX_TID_SAVE_AR_META()` call |

| `dp_rh_rx.c` | Added `DP_RX_TID_SAVE_AR_META()` call |

| `vdrv_dp_if.c` | `vdrv_dp_rx_tid()` reads from ar_meta |

| `vdrv_dp_if.c` | `vdrv_dp_if_wbuf_set_tid()` sets ar_meta first |

| `ar_dp.c` | Uses ar_meta-aware functions |

| `ar_qos.c` | Updates effective TID in ar_meta |

### Migration Notes

**For New Code:**

Use `vdrv_dp_rx_tid()` - it now reads from ar_meta (CB-independent)

Use `vdrv_dp_if_ar_meta_get_tid()` for direct ar_meta access

Use `vdrv_dp_if_ar_meta_set_tid()` to set TID

**For Existing Code:**

No changes required - `vdrv_dp_rx_tid()` is transparently CB-independent

CB is still updated via `wbuf_set_tid()` for backward compatibility

Once fully migrated, CB updates can be removed

**Future Optimization:**

Once all TID reads use ar_meta, remove CB updates:

void vdrv_dp_if_wbuf_set_tid(struct sk_buff* skb, int tid)
{
  /* Set TID in ar_meta only (fully CB-independent) */
  skb->ar_meta.tid = (uint8_t)tid;
  /* wbuf_set_tid(skb, tid);  -- REMOVED: CB no longer needed */
}

──────────────────────────────────────────────────

# Integration Points

## How Integration Points Were Identified

The following methodology was used to identify all relevant integration points:

**Codebase Search**: Used `codebase-retrieval` tool to search for:

- "TID access patterns in wireless driver"

- "QDF_NBUF_CB_RX_TID_VAL usage"

- "skb_clone and skb_copy usage in ar driver"

- "vdrv_dp_rx_tid usage"

**File Analysis**: Examined key files:

- `ar_dp.c` - Main datapath processing

- `ar_qos.c` - QoS priority handling

- `ar_bgmon.c` - Background monitoring

- `ar_dp_if.c` - AR datapath interface

**Grep Patterns**: Searched for patterns:

```bash

grep -r "vdrv_dp_rx_tid" ap/src/wlan-drivers/ar/

grep -r "QDF_NBUF_CB_RX_TID_VAL" ap/src/wlan-drivers/ar/

grep -r "skb_clone" ap/src/wlan-drivers/ar/

grep -r "->tid" ap/src/wlan-drivers/ar/core/

```

## Current Integration Points

### 1. ar_dp_rx_handle() - Main Rx Path

**File:** `ap/src/wlan-drivers/ar/core/src/ar_dp.c`

**Line:** ~2734

AR_STATUS ar_dp_rx_handle(struct sk_buff* skb, struct ar_dp_soc_s* soc, uint8_t vdev_id)
{
  // ... setup code ...

  if (qdf_unlikely(vdev->apc.enable || vdev->dp_pdev->pcap_active ||
                   IEEE80211_IS_MULTICAST(eh->ether_dhost) ||
                   (eh->ether_type == htons(ETHERTYPE_PAE)))) {
    // Rx descriptor allocated for special cases
    ars = ar_dp_alloc_msdu_desc();
    // ...
    tid = ars->tid;                    // TID from descriptor
    peer_id = ars->peer_id;
  } else {
    status = AR_STATUS_SUCCESS;
    tid = vdrv_dp_rx_tid(skb);         // <-- Current TID access
    peer_id = vdrv_dp_rx_peer_id(skb);
  }

  // Later: TID used for QoS processing
  ar_qos_dp_rx_set_prio(skb, vdev, tid);
}

**Potential Enhancement with ar_meta:**

// After getting TID, cache it for later use
tid = vdrv_dp_rx_tid(skb);
vdrv_dp_if_ar_meta_set_tid(skb, tid);  // NEW: Cache TID

// Later in the function or downstream:
// Instead of querying again, use cached value
uint8_t cached_tid = vdrv_dp_if_ar_meta_get_tid(skb);

**Why This Location:**

Entry point for all Rx packets

TID is fetched here and used multiple times downstream

Caching here benefits all subsequent processing

### 2. ar_qos_dp_rx_set_prio() - QoS Priority Setting

**File:** `ap/src/wlan-drivers/ar/core/src/ar_qos.c`

**Line:** 16

void ar_qos_dp_rx_set_prio(struct sk_buff* skb, struct ar_dp_vdev_s* vdev, uint8_t tid)
{
  if (AR_IS_QOS_PRIO_FIXED(vdev)) {
    ar_os_skb_set_priority(skb, WME_AC_TO_TID(AR_GET_QOS_PRIO(vdev)));
  } else {
    AR_CEIL_QOS_TID(vdev, tid);
    ar_os_skb_set_priority(skb, tid);
  }
}

**Potential Enhancement with ar_meta:**

void ar_qos_dp_rx_set_prio(struct sk_buff* skb, struct ar_dp_vdev_s* vdev, uint8_t tid)
{
  // Could alternatively read from ar_meta if tid parameter is uncertain
  // uint8_t tid = vdrv_dp_if_ar_meta_get_tid(skb);

  if (AR_IS_QOS_PRIO_FIXED(vdev)) {
    ar_os_skb_set_priority(skb, WME_AC_TO_TID(AR_GET_QOS_PRIO(vdev)));
  } else {
    AR_CEIL_QOS_TID(vdev, tid);
    ar_os_skb_set_priority(skb, tid);
  }

  // Optionally update ar_meta.tid to reflect final effective TID
  // vdrv_dp_if_ar_meta_set_tid(skb, final_tid);
}

**Why This Location:**

Consumes TID for priority decisions

TID may be modified by ceiling operations

Could cache effective TID for downstream logging/debugging

### 3. ar_qos_dp_set_hs20_qos_map() - Hotspot 2.0 QoS Mapping

**File:** `ap/src/wlan-drivers/ar/core/src/ar_qos.c`

**Line:** 49

void ar_qos_dp_set_hs20_qos_map(struct sk_buff* skb, struct ar_dp_peer_s* peer,
                                struct ar_dp_vdev_s* vdev, int* v_wme_ac, int* v_pri)
{
  // ... DSCP/TID calculation ...

found:
  ac = AR_TID_TO_WME_AC(tid);
  vdrv_dp_if_wbuf_set_tid(skb, tid);   // Sets TID in QDF CB
  skb->priority = ac;
  *v_wme_ac = ac;
  *v_pri = tid;
}

**Potential Enhancement with ar_meta:**

found:
ac = AR_TID_TO_WME_AC(tid);
vdrv_dp_if_wbuf_set_tid(skb, tid);
vdrv_dp_if_ar_meta_set_tid(skb, tid);  // NEW: Cache for consistency
skb->priority = ac;

**Why This Location:**

Sets final TID for Hotspot 2.0 traffic

Downstream code may need to verify TID

ar_meta provides single source of truth

### 4. ar_qos_dp_set_map_pri_fixed() - Fixed Priority Mapping

**File:** `ap/src/wlan-drivers/ar/core/src/ar_qos.c`

**Line:** 108

void ar_qos_dp_set_map_pri_fixed(struct sk_buff* skb, struct ar_dp_vdev_s* vdev,
                                 int* v_wme_ac, int* v_pri)
{
  uint32_t ac = AR_GET_QOS_PRIO(vdev);
  *v_pri = AR_WME_AC_TO_TID(ac);
  *v_wme_ac = ac;
  vdrv_dp_if_wbuf_set_tid(skb, *v_pri);
  skb->priority = ac;
}

**Potential Enhancement with ar_meta:**

void ar_qos_dp_set_map_pri_fixed(struct sk_buff* skb, struct ar_dp_vdev_s* vdev,
                                 int* v_wme_ac, int* v_pri)
{
  uint32_t ac = AR_GET_QOS_PRIO(vdev);
  *v_pri = AR_WME_AC_TO_TID(ac);
  *v_wme_ac = ac;
  vdrv_dp_if_wbuf_set_tid(skb, *v_pri);
  vdrv_dp_if_ar_meta_set_tid(skb, *v_pri);  // NEW: Cache fixed priority TID
  skb->priority = ac;
}

### 5. ar_qos_dp_set_map_dstream_8021p() - 802.1p Priority Mapping

**File:** `ap/src/wlan-drivers/ar/core/src/ar_qos.c`

**Line:** 117

void ar_qos_dp_set_map_dstream_8021p(struct sk_buff* skb, struct ar_dp_vdev_s* vdev,
                                     int* v_wme_ac, int* v_pri)
{
  // ... 802.1p to WMM mapping ...

  *v_wme_ac = AR_TID_TO_WME_AC(tid);
  AR_CEIL_QOS_PRIO(vdev, *v_wme_ac, tid);
  vdrv_dp_if_wbuf_set_tid(skb, tid);
  skb->priority = *v_wme_ac;
  *v_pri = tid;
}

**Why This Location:**

Sets TID based on 802.1p VLAN priority

TID is calculated after complex mapping

Caching prevents recalculation

### 6. ar_qos_dp_set_map_dstream_tos() - ToS/DSCP Mapping

**File:** `ap/src/wlan-drivers/ar/core/src/ar_qos.c`

**Line:** 185

AR_STATUS ar_qos_dp_set_map_dstream_tos(struct sk_buff* skb, struct ar_dp_vdev_s* vdev,
                                        int* v_wme_ac, int* v_pri)
{
  // ... IP ToS to TID mapping ...

  *v_wme_ac = AR_TID_TO_WME_AC(pri);
  AR_CEIL_QOS_PRIO(vdev, *v_wme_ac, pri);
  vdrv_dp_if_wbuf_set_tid(skb, pri);
  skb->priority = *v_wme_ac;
  *v_pri = pri;
  return AR_STATUS_SUCCESS;
}

### 7. ar_qos_dp_set_map_dstream_dscp() - DSCP Mapping

**File:** `ap/src/wlan-drivers/ar/core/src/ar_qos.c`

**Line:** 222

AR_STATUS ar_qos_dp_set_map_dstream_dscp(struct sk_buff* skb, struct ar_dp_vdev_s* vdev,
                                         int* v_wme_ac, int* v_pri)
{
  // ... DSCP to TID mapping with special handling for DSCP 46 ...

  AR_CEIL_QOS_PRIO(vdev, wme_ac, pri);
  vdrv_dp_if_wbuf_set_tid(skb, pri);
  skb->priority = wme_ac;
  *v_pri = pri;
  *v_wme_ac = wme_ac;
  return AR_STATUS_SUCCESS;
}

## SKB Clone/Copy Locations

These locations are important because ar_meta must be preserved during cloning:

### 1. ar_bgmon.c - Background Monitoring

**File:** `ap/src/wlan-drivers/ar/core/src/ar_bgmon.c`

Background monitoring uses `skb_clone()` to create copies of packets for analysis.

With the kernel patch, `ar_meta` is automatically copied during clone operations.

// Example pattern in ar_bgmon.c
struct sk_buff* clone = skb_clone(skb, GFP_ATOMIC);
if (clone) {
  // ar_meta is automatically preserved due to kernel patch
  // clone->ar_meta.tid == skb->ar_meta.tid
  process_background_packet(clone);
}

## Summary Table: All TID Access Points

| File | Function | TID Access Pattern | ar_meta Applicable |

|------|----------|-------------------|-------------------|

| ar_dp.c | ar_dp_rx_handle() | vdrv_dp_rx_tid() | Yes - Cache source |

| ar_qos.c | ar_qos_dp_rx_set_prio() | Parameter | Yes - Verify/Update |

| ar_qos.c | ar_qos_dp_set_hs20_qos_map() | Calculated | Yes - Cache result |

| ar_qos.c | ar_qos_dp_set_map_pri_fixed() | Calculated | Yes - Cache result |

| ar_qos.c | ar_qos_dp_set_map_dstream_8021p() | Calculated | Yes - Cache result |

| ar_qos.c | ar_qos_dp_set_map_dstream_tos() | Calculated | Yes - Cache result |

| ar_qos.c | ar_qos_dp_set_map_dstream_dscp() | Calculated | Yes - Cache result |

──────────────────────────────────────────────────

# Data Flow Analysis

## Receive (Rx) Path Data Flow

┌─────────────────────────────────────────────────────────────────────────┐
│                    Rx Path ar_meta Data Flow                            │
├─────────────────────────────────────────────────────────────────────────┤
│                                                                         │
│  Step 1: Packet Received from Hardware                                  │
│  ─────────────────────────────────────                                  │
│  ┌─────────────────┐                                                    │
│  │   Hardware     │    Rx Descriptor contains:                          │
│  │   Rx Ring      │    - TID (from 802.11 QoS header)                   │
│  │                │    - Peer ID                                        │
│  │                │    - Rate info (MCS, NSS, BW)                       │
│  └────────┬────────┘                                                    │
│           │                                                             │
│           ▼                                                             │
│  Step 2: SKB Allocated, ar_meta Initialized to 0                        │
│  ────────────────────────────────────────────                           │
│  ┌─────────────────┐   Kernel __build_skb() sets:                       │
│  │   sk_buff       │   skb->ar_meta.tid = 0                             │
│  │   ar_meta:      │   skb->ar_meta.reserve = 0                         │
│  │   tid=0         │                                                    │
│  │   reserve=0     │                                                    │
│  └────────┬────────┘                                                    │
│           │                                                             │
│           ▼                                                             │
│  Step 3: Vendor Driver Fills QDF Control Block                          │
│  ──────────────────────────────────────────────                         │
│  ┌─────────────────┐   QDF_NBUF_CB_RX_TID_VAL(skb) = tid_from_hw        │
│  │   QCA Vendor   │   (TID stored in skb->cb[] array)                   │
│  │   Driver       │                                                     │
│  └────────┬────────┘                                                    │
│           │                                                             │
│           ▼                                                             │
│  Step 4: VDRV Layer - TID Access Point                                  │
│  ──────────────────────────────────────                                 │
│  ┌─────────────────┐                                                    │
│  │   vdrv_dp_if   │   tid = vdrv_dp_rx_tid(skb);                        │
│  │                │   // OPPORTUNITY: Cache TID here                    │
│  │                │   // vdrv_dp_if_ar_meta_set_tid(skb, tid);          │
│  └────────┬────────┘                                                    │
│           │                                                             │
│           ▼                                                             │
│  Step 5: AR Core Rx Handler                                             │
│  ───────────────────────────                                            │
│  ┌─────────────────┐   ar_dp_rx_handle()                                │
│  │   ar_dp.c      │   - Fetches TID via vdrv_dp_rx_tid()                │
│  │                │   - Passes TID to QoS processing                    │
│  └────────┬────────┘                                                    │
│           │                                                             │
│           ▼                                                             │
│  Step 6: QoS Processing                                                 │
│  ───────────────────────                                                │
│  ┌─────────────────┐   ar_qos_dp_rx_set_prio()                          │
│  │   ar_qos.c     │   - Sets skb->priority based on TID                 │
│  │                │   - May apply ceiling to TID                        │
│  └────────┬────────┘                                                    │
│           │                                                             │
│           ▼                                                             │
│  Step 7: Network Stack Delivery                                         │
│  ───────────────────────────────                                        │
│  ┌─────────────────┐                                                    │
│  │   Linux        │   netif_receive_skb() / netif_rx()                  │
│  │   Net Stack    │   ar_meta still available if needed                 │
│  └─────────────────┘                                                    │
│                                                                         │
└─────────────────────────────────────────────────────────────────────────┘

## Transmit (Tx) Path Data Flow

┌─────────────────────────────────────────────────────────────────────────┐
│                    Tx Path ar_meta Data Flow                            │
├─────────────────────────────────────────────────────────────────────────┤
│                                                                         │
│  Step 1: Packet from Network Stack                                      │
│  ──────────────────────────────────                                     │
│  ┌─────────────────┐                                                    │
│  │   Linux        │   dev_queue_xmit() called                           │
│  │   Net Stack    │   skb->priority set from socket/TC                  │
│  │                │   ar_meta.tid = 0 (not yet set)                     │
│  └────────┬────────┘                                                    │
│           │                                                             │
│           ▼                                                             │
│  Step 2: QoS Classification                                             │
│  ───────────────────────────                                            │
│  ┌─────────────────┐   Determines TID based on:                         │
│  │   ar_qos.c     │   - Fixed priority (vdev config)                    │
│  │   functions    │   - 802.1p VLAN priority                            │
│  │                │   - IP ToS/DSCP                                     │
│  │                │   - Hotspot 2.0 QoS map                             │
│  └────────┬────────┘                                                    │
│           │                                                             │
│           │   vdrv_dp_if_wbuf_set_tid(skb, tid);                        │
│           │   // OPPORTUNITY: Also set ar_meta                          │
│           │   // vdrv_dp_if_ar_meta_set_tid(skb, tid);                  │
│           ▼                                                             │
│  Step 3: AR Core Tx Handler                                             │
│  ───────────────────────────                                            │
│  ┌─────────────────┐   ar_dp_tx_handle()                                │
│  │   ar_dp.c      │   - Processes packet for transmission               │
│  │                │   - May need TID for queue selection                │
│  └────────┬────────┘                                                    │
│           │                                                             │
│           ▼                                                             │
│  Step 4: VDRV Layer to Vendor Driver                                    │
│  ────────────────────────────────────                                   │
│  ┌─────────────────┐   vdrv_dp_if_tx_send()                             │
│  │   vdrv_dp_if   │   - TID available from ar_meta if cached            │
│  │                │   - Or via vdrv_dp_if_wbuf_get_tid()                │
│  └────────┬────────┘                                                    │
│           │                                                             │
│           ▼                                                             │
│  Step 5: Hardware Tx                                                    │
│  ────────────────────                                                   │
│  ┌─────────────────┐                                                    │
│  │   QCA Driver   │   Packet queued to hardware Tx ring                 │
│  │   + Hardware   │   TID determines WMM access category queue          │
│  └─────────────────┘                                                    │
│                                                                         │
└─────────────────────────────────────────────────────────────────────────┘

## SKB Lifecycle and ar_meta

┌─────────────────────────────────────────────────────────────────────────┐
│                  SKB Lifecycle with ar_meta                             │
├─────────────────────────────────────────────────────────────────────────┤
│                                                                         │
│  1. ALLOCATION                                                          │
│     __build_skb() / __alloc_skb()                                       │
│     └─► ar_meta initialized to {tid=0, reserve=0}                       │
│                                                                         │
│  2. POPULATION                                                          │
│     Driver receives packet, fills data                                  │
│     └─► ar_meta can be set via vdrv_dp_if_ar_meta_set_*()               │
│                                                                         │
│  3. CLONE (if needed)                                                   │
│     skb_clone() / __skb_clone()                                         │
│     └─► ar_meta COPIED to clone (C(ar_meta) in kernel)                  │
│                                                                         │
│  4. COPY (if needed)                                                    │
│     skb_copy() / __skb_copy()                                           │
│     └─► ar_meta copied with full skb copy                               │
│                                                                         │
│  5. PROCESSING                                                          │
│     Multiple driver layers access packet                                │
│     └─► ar_meta accessible via vdrv_dp_if_ar_meta_get_*()               │
│                                                                         │
│  6. FREE                                                                │
│     kfree_skb() / consume_skb()                                         │
│     └─► ar_meta freed with skb                                          │
│                                                                         │
│  7. RECYCLE (if recycler enabled)                                       │
│     skb_recycler_clear_flags()                                          │
│     └─► ar_meta CLEARED to {tid=0, reserve=0}                           │
│                                                                         │
└─────────────────────────────────────────────────────────────────────────┘

──────────────────────────────────────────────────

# API Reference

## Complete Function Reference

### vdrv_dp_if_ar_meta_get_tid()

/**
 * @brief Get cached TID value from sk_buff ar_meta
 *
 * Retrieves the Traffic Identifier cached in the sk_buff's ar_meta
 * structure. This is more efficient than querying the QDF control
 * block or hardware descriptors for frequently accessed TID values.
 *
 * @param skb Pointer to the socket buffer
 *
 * @return uint8_t The cached TID value (0-15)
 *
 * @note Returns 0 if ar_meta has not been explicitly set
 * @note Thread-safe following sk_buff locking conventions
 *
 * @see vdrv_dp_if_ar_meta_set_tid()
 * @see vdrv_dp_rx_tid()
 */
uint8_t vdrv_dp_if_ar_meta_get_tid(struct sk_buff* skb);

**Implementation:**

uint8_t vdrv_dp_if_ar_meta_get_tid(struct sk_buff* skb)
{
  return skb->ar_meta.tid;
}

**Usage Notes:**

Always check that skb is not NULL before calling

Returns 0 for newly allocated skbs (before TID is cached)

Value persists through skb_clone() operations

──────────────────────────────────────────────────

### vdrv_dp_if_ar_meta_set_tid()

/**
 * @brief Set TID value in sk_buff ar_meta cache
 *
 * Caches the Traffic Identifier in the sk_buff's ar_meta structure
 * for efficient later retrieval. This should be called once when
 * TID is first determined, typically early in Rx processing.
 *
 * @param skb Pointer to the socket buffer
 * @param tid Traffic Identifier to cache (0-15)
 *
 * @return void
 *
 * @note TID values > 15 are technically valid but unusual
 * @note Overwrites any previously cached TID value
 *
 * @see vdrv_dp_if_ar_meta_get_tid()
 * @see vdrv_dp_rx_tid()
 */
void vdrv_dp_if_ar_meta_set_tid(struct sk_buff* skb, uint8_t tid);

**Implementation:**

void vdrv_dp_if_ar_meta_set_tid(struct sk_buff* skb, uint8_t tid)
{
  skb->ar_meta.tid = tid;
}

**Usage Notes:**

Best practice: Call immediately after TID is determined

The tid parameter is 8 bits; values 0-15 are standard WMM TIDs

Setting TID does not affect QDF control block or hardware

──────────────────────────────────────────────────

### vdrv_dp_if_ar_meta_get_reserve()

/**
 * @brief Get reserved field value from sk_buff ar_meta
 *
 * Retrieves the reserved field from the sk_buff's ar_meta structure.
 * This field is available for future Arista-specific metadata.
 *
 * @param skb Pointer to the socket buffer
 *
 * @return uint8_t The reserved field value
 *
 * @note Currently unused; reserved for future extensions
 * @note Returns 0 for newly allocated skbs
 *
 * @see vdrv_dp_if_ar_meta_set_reserve()
 */
uint8_t vdrv_dp_if_ar_meta_get_reserve(struct sk_buff* skb);

**Implementation:**

uint8_t vdrv_dp_if_ar_meta_get_reserve(struct sk_buff* skb)
{
  return skb->ar_meta.reserve;
}

**Future Use Cases:**

Packet classification flags

Processing stage indicators

Debug/trace markers

Feature-specific metadata

──────────────────────────────────────────────────

### vdrv_dp_if_ar_meta_set_reserve()

/**
 * @brief Set reserved field value in sk_buff ar_meta cache
 *
 * Sets the reserved field in the sk_buff's ar_meta structure.
 * This field is available for future Arista-specific metadata.
 *
 * @param skb Pointer to the socket buffer
 * @param reserve Value to set in reserved field
 *
 * @return void
 *
 * @note Currently unused; reserved for future extensions
 *
 * @see vdrv_dp_if_ar_meta_get_reserve()
 */
void vdrv_dp_if_ar_meta_set_reserve(struct sk_buff* skb, uint8_t reserve);

**Implementation:**

void vdrv_dp_if_ar_meta_set_reserve(struct sk_buff* skb, uint8_t reserve)
{
  skb->ar_meta.reserve = reserve;
}

──────────────────────────────────────────────────

### vdrv_dp_if_ar_meta_clear()

/**
 * @brief Clear ar_meta cache in sk_buff
 *
 * Resets both tid and reserve fields to 0. Useful when reusing
 * an skb for a different packet or when explicit clearing is needed.
 *
 * @param skb Pointer to the socket buffer
 *
 * @return void
 *
 * @note Automatically called by kernel during skb recycling
 * @note Call this if manually resetting an skb for reuse
 *
 * @see vdrv_dp_if_ar_meta_set_tid()
 * @see vdrv_dp_if_ar_meta_set_reserve()
 */
void vdrv_dp_if_ar_meta_clear(struct sk_buff* skb);

**Implementation:**

void vdrv_dp_if_ar_meta_clear(struct sk_buff* skb)
{
  skb->ar_meta.tid = 0;
  skb->ar_meta.reserve = 0;
}

**When to Use:**

Before reusing an skb for a different packet

When explicitly resetting packet metadata

During error recovery paths

──────────────────────────────────────────────────

# Usage Examples

## Example 1: Basic TID Caching in Rx Path

#include <vdrv_dp_if.h>

/**
 * Process received packet with ar_meta TID caching
 */
AR_STATUS process_rx_packet_with_caching(struct sk_buff* skb,
                                         struct ar_dp_soc_s* soc,
                                         uint8_t vdev_id)
{
  uint8_t tid;
  struct ar_dp_vdev_s* vdev;

  /* Get vdev context */
  vdev = ar_dp_get_vdev_by_id(soc, vdev_id);
  if (!vdev) {
    return AR_STATUS_E_INVAL;
  }

  /* Fetch TID from QDF control block (populated by vendor driver) */
  tid = vdrv_dp_rx_tid(skb);

  /* Cache TID in ar_meta for efficient subsequent access */
  vdrv_dp_if_ar_meta_set_tid(skb, tid);

  /* ... continue with packet processing ... */

  /* Later in processing, use cached TID instead of re-querying */
  tid = vdrv_dp_if_ar_meta_get_tid(skb);
  ar_qos_dp_rx_set_prio(skb, vdev, tid);

  ar_dp_vdev_release_ref(vdev);
  return AR_STATUS_SUCCESS;
}

## Example 2: TID Preservation Across Clone

#include <vdrv_dp_if.h>

/**
 * Clone packet while preserving ar_meta
 *
 * The kernel patch ensures ar_meta is automatically copied
 * during skb_clone(), but this example shows how to verify.
 */
void clone_with_metadata_check(struct sk_buff* skb)
{
  struct sk_buff* clone;
  uint8_t original_tid, clone_tid;

  /* Set TID in original skb */
  original_tid = vdrv_dp_rx_tid(skb);
  vdrv_dp_if_ar_meta_set_tid(skb, original_tid);

  /* Clone the skb */
  clone = skb_clone(skb, GFP_ATOMIC);
  if (!clone) {
    ar_os_pr_err("Failed to clone skb");
    return;
  }

  /* Verify ar_meta was copied (kernel patch ensures this) */
  clone_tid = vdrv_dp_if_ar_meta_get_tid(clone);

  /* Debug: should be true due to kernel patch */
  if (clone_tid != original_tid) {
    ar_os_pr_warn("ar_meta TID not preserved in clone: orig=%u, clone=%u",
                  original_tid, clone_tid);
  }

  /* Process clone */
  process_cloned_packet(clone);

  kfree_skb(clone);
}

## Example 3: Using Reserve Field for Debugging

#include <vdrv_dp_if.h>

/* Define debug markers for reserve field */
#define AR_META_STAGE_RX_ENTRY    0x01
#define AR_META_STAGE_QOS_DONE    0x02
#define AR_META_STAGE_ACL_DONE    0x04
#define AR_META_STAGE_DELIVERED   0x08

/**
 * Track packet processing stages using reserve field
 */
void mark_processing_stage(struct sk_buff* skb, uint8_t stage)
{
  uint8_t current = vdrv_dp_if_ar_meta_get_reserve(skb);
  vdrv_dp_if_ar_meta_set_reserve(skb, current | stage);
}

/**
 * Check if packet passed through specific stage
 */
bool packet_passed_stage(struct sk_buff* skb, uint8_t stage)
{
  return (vdrv_dp_if_ar_meta_get_reserve(skb) & stage) != 0;
}

/**
 * Example usage in Rx path
 */
void rx_with_stage_tracking(struct sk_buff* skb)
{
  /* Mark entry */
  mark_processing_stage(skb, AR_META_STAGE_RX_ENTRY);

  /* Do QoS processing */
  process_qos(skb);
  mark_processing_stage(skb, AR_META_STAGE_QOS_DONE);

  /* Do ACL processing */
  process_acl(skb);
  mark_processing_stage(skb, AR_META_STAGE_ACL_DONE);

  /* On error, can check what stages completed */
  if (error_condition) {
    uint8_t stages = vdrv_dp_if_ar_meta_get_reserve(skb);
    ar_os_pr_err("Packet failed after stages: 0x%02x", stages);
  }
}

## Example 4: Integration with QoS Functions

#include <vdrv_dp_if.h>
#include "ar_qos.h"

/**
 * Enhanced QoS mapping that uses ar_meta for consistency
 */
void ar_qos_dp_set_map_enhanced(struct sk_buff* skb,
                                struct ar_dp_vdev_s* vdev,
                                int* v_wme_ac,
                                int* v_pri)
{
  int tid;
  int wme_ac;

  /* Perform DSCP-based TID calculation */
  /* ... (existing logic) ... */
  tid = calculated_tid;

  /* Set TID in QDF control block (existing mechanism) */
  vdrv_dp_if_wbuf_set_tid(skb, tid);

  /* ALSO cache in ar_meta for unified access */
  vdrv_dp_if_ar_meta_set_tid(skb, tid);

  /* Calculate WMM access category */
  wme_ac = AR_TID_TO_WME_AC(tid);

  /* Apply ceiling if configured */
  AR_CEIL_QOS_PRIO(vdev, wme_ac, tid);

  /* Update ar_meta with final (possibly adjusted) TID */
  vdrv_dp_if_ar_meta_set_tid(skb, tid);

  skb->priority = wme_ac;
  *v_wme_ac = wme_ac;
  *v_pri = tid;
}

## Example 5: Error Handling Pattern

#include <vdrv_dp_if.h>

/**
 * Safe ar_meta access with NULL checks
 */
static inline uint8_t safe_get_cached_tid(struct sk_buff* skb)
{
  if (unlikely(!skb)) {
    ar_os_pr_warn("safe_get_cached_tid: NULL skb");
    return 0;
  }
  return vdrv_dp_if_ar_meta_get_tid(skb);
}

static inline void safe_set_cached_tid(struct sk_buff* skb, uint8_t tid)
{
  if (unlikely(!skb)) {
    ar_os_pr_warn("safe_set_cached_tid: NULL skb");
    return;
  }
  if (unlikely(tid > 15)) {
    ar_os_pr_warn("safe_set_cached_tid: TID %u > 15", tid);
  }
  vdrv_dp_if_ar_meta_set_tid(skb, tid);
}

──────────────────────────────────────────────────

# Performance Considerations

## Memory Overhead

| Aspect | Impact | Notes |

|--------|--------|-------|

| sk_buff size increase | +2 bytes | tid (1 byte) + reserve (1 byte) |

| Per-packet overhead | Negligible | 2 bytes per sk_buff in pool |

| Cache line impact | Minimal | Fields adjacent to existing ar fields |

| Total system impact | ~few KB | Depends on sk_buff pool size |

## Access Time Comparison

┌─────────────────────────────────────────────────────────────────────────┐
│              TID Access Method Comparison                               │
├────────────────────────┬────────────┬───────────────────────────────────┤
│ Method                 │ Operations │ Notes                             │
├────────────────────────┼────────────┼───────────────────────────────────┤
│ ar_meta cache          │ 1 load     │ Direct struct member access       │
│ skb->ar_meta.tid       │            │ Best: L1 cache likely hot         │
├────────────────────────┼────────────┼───────────────────────────────────┤
│ QDF Control Block      │ 2-3 loads  │ cb array offset + extraction      │
│ QDF_NBUF_CB_RX_TID_VAL │ + macro    │ Macro expansion overhead          │
├────────────────────────┼────────────┼───────────────────────────────────┤
│ Hardware Descriptor    │ Function   │ May cause cache miss              │
│ vdrv_dp_if_rx_msdu_*   │ call + I/O │ Worst: potential MMIO             │
└────────────────────────┴────────────┴───────────────────────────────────┘

## CPU Cycle Estimates (Approximate)

| Operation | Cycles | Condition |

|-----------|--------|-----------|

| ar_meta read (L1 hit) | 3-4 | Common case |

| ar_meta read (L2 hit) | 10-15 | Less common |

| QDF CB access | 10-20 | Macro + indirection |

| HW descriptor query | 50-200+ | Depends on bus |

## Throughput Impact

For a system processing 100,000 packets/second:

Without ar_meta caching (3 TID lookups per packet via QDF CB):
= 100,000 * 3 * 15 cycles = 4,500,000 cycles/second

With ar_meta caching (1 QDF lookup + 2 ar_meta reads):
= 100,000 * (15 + 2*4) cycles = 2,300,000 cycles/second

Savings: ~2.2 million cycles/second = ~1ms of CPU time per second

## Best Practices for Performance

**Cache Early**: Set ar_meta.tid as soon as TID is determined

**Read Often**: Use ar_meta for all subsequent TID reads

**Avoid Redundant Sets**: Don't set ar_meta if value hasn't changed

**Batch Operations**: If setting both tid and reserve, consider a combined function

/* Good: Cache once, read multiple times */
tid = vdrv_dp_rx_tid(skb);
vdrv_dp_if_ar_meta_set_tid(skb, tid);
// ... later ...
tid = vdrv_dp_if_ar_meta_get_tid(skb);  // Fast
tid = vdrv_dp_if_ar_meta_get_tid(skb);  // Fast

/* Bad: Re-querying each time */
tid = vdrv_dp_rx_tid(skb);  // Slow
tid = vdrv_dp_rx_tid(skb);  // Slow again

──────────────────────────────────────────────────

# Future Extensions

## Using the Reserve Field

The 8-bit `reserve` field is available for future Arista-specific features.

Here are potential use cases:

### Option 1: Packet Classification Flags

/* Bit definitions for reserve field */
#define AR_META_F_MULTICAST    0x01   /* Packet is multicast */
#define AR_META_F_ENCRYPTED    0x02   /* Packet was encrypted */
#define AR_META_F_FRAGMENT     0x04   /* Packet is a fragment */
#define AR_META_F_AMSDU        0x08   /* Packet is part of A-MSDU */
#define AR_META_F_AMPDU        0x10   /* Packet is part of A-MPDU */
#define AR_META_F_EAPOL        0x20   /* Packet is EAPOL frame */
#define AR_META_F_DHCP         0x40   /* Packet is DHCP */
#define AR_META_F_ARP          0x80   /* Packet is ARP */

/* Helper macros */
#define AR_META_SET_FLAG(skb, f)   \
vdrv_dp_if_ar_meta_set_reserve(skb, \
                               vdrv_dp_if_ar_meta_get_reserve(skb) | (f))

#define AR_META_CLR_FLAG(skb, f)   \
vdrv_dp_if_ar_meta_set_reserve(skb, \
                               vdrv_dp_if_ar_meta_get_reserve(skb) & ~(f))

#define AR_META_TST_FLAG(skb, f)   \
(vdrv_dp_if_ar_meta_get_reserve(skb) & (f))

### Option 2: Processing State Machine

/* State values for reserve field */
#define AR_META_STATE_INIT        0x00
#define AR_META_STATE_CLASSIFYING 0x01
#define AR_META_STATE_QOS_DONE    0x02
#define AR_META_STATE_ACL_DONE    0x03
#define AR_META_STATE_READY       0x04
#define AR_META_STATE_ERROR       0xFF

### Option 3: Link/Radio Index

/* For multi-link operation (802.11be MLO) */
/* Lower 4 bits: Link ID (0-15) */
/* Upper 4 bits: Radio index (0-15) */
#define AR_META_LINK_ID(r)     ((r) & 0x0F)
#define AR_META_RADIO_IDX(r)   (((r) >> 4) & 0x0F)
#define AR_META_MAKE_LR(l, r)  (((r) << 4) | ((l) & 0x0F))

## Expanding ar_meta Structure

If more fields are needed in the future, the ar_meta structure in the

kernel patch can be expanded:

/* Current (2 bytes) */
struct {
  __u8    tid;
  __u8    reserve;
} ar_meta;

/* Future expansion example (4 bytes) */
struct {
  __u8    tid;
  __u8    flags;
  __u8    link_id;
  __u8    radio_idx;
} ar_meta;

**Considerations for expansion:**

sk_buff size impact

Structure alignment

Backward compatibility with existing drivers

Kernel version dependencies

──────────────────────────────────────────────────

# Related Files

## Complete File Reference

### Kernel Patch Files

| File | Description |

|------|-------------|

| `ap/platform/patches/kernel/5.4/12.5/common/ar_meta_cache.patch` | Main patch adding ar_meta to sk_buff |

### VDRV Interface Layer

| File | Description |

|------|-------------|

| `ap/src/wlan-drivers/ar/vdrv_if/inc/vdrv_dp_if.h` | Header with ar_meta function declarations |

| `ap/src/wlan-drivers/ar/vdrv_if/qca/common/vdrv_dp_if.c` | Implementation of ar_meta accessor functions |

### Core Driver Files (Potential Integration Points)

| File | Description |

|------|-------------|

| `ap/src/wlan-drivers/ar/core/src/ar_dp.c` | Main datapath processing |

| `ap/src/wlan-drivers/ar/core/src/ar_qos.c` | QoS priority handling |

| `ap/src/wlan-drivers/ar/core/src/ar_bgmon.c` | Background monitoring |

| `ap/src/wlan-drivers/ar/core/src/ar_dp.h` | Datapath header |

| `ap/src/wlan-drivers/ar/core/src/ar_qos.h` | QoS header |

### AR Interface Layer

| File | Description |

|------|-------------|

| `ap/src/wlan-drivers/ar/ar_if/qca/common/ar_dp_if.c` | AR datapath interface |

### Documentation

| File | Description |

|------|-------------|

| `ap/src/wlan-drivers/ar/vdrv_if/doc/ar_meta_cache.md` | This documentation file |

## File Dependency Graph

ar_meta_cache.patch
         │
         ▼
┌─────────────────┐
│  Linux Kernel   │
│  (sk_buff.h)    │
└────────┬────────┘
         │
┌────────┼────────────────────────┐
│        │                        │
▼        ▼                        ▼
┌────────────────┐  ┌────────────┐  ┌────────────────┐
│ vdrv_dp_if.h   │  │ QCA Vendor │  │ Other Vendors  │
│ (declarations) │  │ Driver     │  │ (future)       │
└────────┬───────┘  └──────┬─────┘  └────────────────┘
         │                 │
         ▼                 ▼
┌────────────────┐  ┌────────────────┐
│ vdrv_dp_if.c   │  │                │
│ (QCA impl)     │  │  Includes      │
└────────┬───────┘  │  vdrv_dp_if.h  │
         │          └────────────────┘
         ▼
┌───────────────────────────────────────────────┐
│                 AR Core Driver                │
│  ┌───────────┐  ┌───────────┐  ┌────────────┐ │
│  │  ar_dp.c  │  │ ar_qos.c  │  │ ar_bgmon.c │ │
│  └───────────┘  └───────────┘  └────────────┘ │
└───────────────────────────────────────────────┘

──────────────────────────────────────────────────

# Appendix

## A. Code Discovery Methodology

This section documents the methodology used to identify all relevant files and

integration points for the ar_meta cache feature.

### Step 1: Locate the Patch File

# Initial search for patch file
find . -name "*.patch" -o -name "*ar_meta*" 2>/dev/null | head -50

# More specific search
find . -name "*ar_meta_cache*" -o -name "*meta_cache*" 2>/dev/null

# Result: ap/platform/patches/kernel/5.4/12.5/common/ar_meta_cache.patch

### Step 2: Analyze the Patch

# View patch contents
cat ap/platform/patches/kernel/5.4/12.5/common/ar_meta_cache.patch

# Key findings:
# - Adds ar_meta struct to sk_buff
# - Modifies: skbuff.h, skbuff.c, skbuff_recycle.c
# - Fields: tid (u8), reserve (u8)

### Step 3: Find VDRV Interface Files

# List VDRV directory structure
ls -la ap/src/wlan-drivers/ar/vdrv_if/

# Find datapath interface files
find ap/src/wlan-drivers/ar/vdrv_if -name "*dp*"

# Result:
# - ap/src/wlan-drivers/ar/vdrv_if/inc/vdrv_dp_if.h
# - ap/src/wlan-drivers/ar/vdrv_if/qca/common/vdrv_dp_if.c

### Step 4: Search for TID Usage

# Find all TID-related code in AR driver
grep -rn "tid" ap/src/wlan-drivers/ar/core/src/*.c | head -50

# Find QDF TID macro usage
grep -rn "QDF_NBUF_CB_RX_TID_VAL" ap/src/wlan-drivers/

# Find vdrv TID functions
grep -rn "vdrv_dp_rx_tid\|vdrv_dp_if_wbuf_set_tid" ap/src/wlan-drivers/ar/

### Step 5: Identify Clone/Copy Locations

# Find skb_clone usage
grep -rn "skb_clone" ap/src/wlan-drivers/ar/

# Find skb_copy usage
grep -rn "skb_copy" ap/src/wlan-drivers/ar/

# Results in ar_bgmon.c and other files

### Step 6: Use Codebase Retrieval Tool

Queries used with the codebase-retrieval tool:

"TID access patterns in wireless driver ar_dp.c"

"QDF_NBUF_CB_RX_TID_VAL macro usage"

"skb_clone and skb_copy usage in ar driver"

"vdrv_dp_rx_tid function implementation"

"ar_qos TID handling and priority mapping"

## B. Patch Application Instructions

To apply the ar_meta_cache.patch to a kernel tree:

# Navigate to kernel source
cd /path/to/kernel/source

# Apply patch
patch -p1 < /path/to/ar_meta_cache.patch

# Verify changes
git diff include/linux/skbuff.h
git diff net/core/skbuff.c
git diff net/core/skbuff_recycle.c

# Build kernel
make -j$(nproc)

## C. Testing Recommendations

### Unit Tests for VDRV Functions

#include <linux/skbuff.h>
#include <vdrv_dp_if.h>

void test_ar_meta_basic(void)
{
  struct sk_buff* skb = alloc_skb(256, GFP_KERNEL);

  /* Test initial state */
  ASSERT_EQ(vdrv_dp_if_ar_meta_get_tid(skb), 0);
  ASSERT_EQ(vdrv_dp_if_ar_meta_get_reserve(skb), 0);

  /* Test set/get TID */
  vdrv_dp_if_ar_meta_set_tid(skb, 7);
  ASSERT_EQ(vdrv_dp_if_ar_meta_get_tid(skb), 7);

  /* Test set/get reserve */
  vdrv_dp_if_ar_meta_set_reserve(skb, 0xAB);
  ASSERT_EQ(vdrv_dp_if_ar_meta_get_reserve(skb), 0xAB);

  /* Test clear */
  vdrv_dp_if_ar_meta_clear(skb);
  ASSERT_EQ(vdrv_dp_if_ar_meta_get_tid(skb), 0);
  ASSERT_EQ(vdrv_dp_if_ar_meta_get_reserve(skb), 0);

  kfree_skb(skb);
}

void test_ar_meta_clone_preservation(void)
{
  struct sk_buff* skb = alloc_skb(256, GFP_KERNEL);
  struct sk_buff* clone;

  /* Set values in original */
  vdrv_dp_if_ar_meta_set_tid(skb, 5);
  vdrv_dp_if_ar_meta_set_reserve(skb, 0x12);

  /* Clone */
  clone = skb_clone(skb, GFP_KERNEL);

  /* Verify preservation */
  ASSERT_EQ(vdrv_dp_if_ar_meta_get_tid(clone), 5);
  ASSERT_EQ(vdrv_dp_if_ar_meta_get_reserve(clone), 0x12);

  kfree_skb(clone);
  kfree_skb(skb);
}

void test_ar_meta_boundary_values(void)
{
  struct sk_buff* skb = alloc_skb(256, GFP_KERNEL);

  /* Test TID boundary (0-255, but typically 0-15) */
  vdrv_dp_if_ar_meta_set_tid(skb, 0);
  ASSERT_EQ(vdrv_dp_if_ar_meta_get_tid(skb), 0);

  vdrv_dp_if_ar_meta_set_tid(skb, 15);
  ASSERT_EQ(vdrv_dp_if_ar_meta_get_tid(skb), 15);

  vdrv_dp_if_ar_meta_set_tid(skb, 255);
  ASSERT_EQ(vdrv_dp_if_ar_meta_get_tid(skb), 255);

  /* Test reserve boundary */
  vdrv_dp_if_ar_meta_set_reserve(skb, 0);
  ASSERT_EQ(vdrv_dp_if_ar_meta_get_reserve(skb), 0);

  vdrv_dp_if_ar_meta_set_reserve(skb, 255);
  ASSERT_EQ(vdrv_dp_if_ar_meta_get_reserve(skb), 255);

  kfree_skb(skb);
}

## D. Troubleshooting Guide

| Issue | Possible Cause | Solution |

|-------|---------------|----------|

| ar_meta.tid always 0 | TID never cached | Ensure vdrv_dp_if_ar_meta_set_tid() is called |

| TID not preserved in clone | Kernel patch not applied | Verify patch is in kernel build |

| Stale TID after recycle | Recycler not clearing | Check skb_recycler_clear_flags() patch |

| Build fails on ar_meta | Kernel headers mismatch | Rebuild with patched kernel headers |

## E. Version History

| Version | Date | Author | Changes |

|---------|------|--------|---------|

| 1.0 | 2026-01 | AR Team | Initial implementation |

## F. Glossary

| Term | Definition |

|------|------------|

| **sk_buff** | Socket buffer - Linux kernel structure for network packet handling |

| **TID** | Traffic Identifier - 802.11 QoS priority value (0-15) |

| **WMM** | Wi-Fi Multimedia - QoS certification for wireless networks |

| **AC** | Access Category - WMM queue priority (VO, VI, BE, BK) |

| **VDRV** | Vendor Driver Interface - Arista abstraction layer |

| **QDF** | QCA Driver Framework - Qualcomm's driver infrastructure |

| **DSCP** | Differentiated Services Code Point - IP QoS marking |

| **A-MSDU** | Aggregated MAC Service Data Unit |

| **A-MPDU** | Aggregated MAC Protocol Data Unit |

| **MLO** | Multi-Link Operation - 802.11be feature |

──────────────────────────────────────────────────

*End of Documentation*

*Document generated: 2026-01*

*Total lines: ~1050*